PXELINUX

A bootloader for Linux using the PXE network booting protocol

Copyright (C) 1994-2004 H. Peter Anvin

This program is provided under the terms of the GNU General Public

License, version 2 or, at your option, any later version.  There is no

warranty, neither expressed nor implied, to the function of this

program.  Please see the included file COPYING for details.

----------------------------------------------------------------------

PXELINUX is a SYSLINUX derivative, for booting Linux off a network

server, using a network ROM conforming to the Intel PXE (Pre-Execution

Environment) specification.  PXELINUX is *not* a program that is

intended to be flashed or burned into a PROM on the network card; if

you want that, check out NILO (http://www.nilo.org/).  NILO can also

be used to create a PXE-compliant boot PROM for most network cards

which have drivers for Linux or FreeBSD.

++++ HOW TO CONFIGURE PXELINUX ++++

PXELINUX operates in many ways like SYSLINUX.  If you are not familiar

with SYSLINUX, read syslinux.doc first, since this documentation only

explains the differences.

On the TFTP server, create the directory "/tftpboot", and copy the

following files to it:

pxelinux.0		- from the SYSLINUX distribution

any kernel or initrd images you want to boot

Finally, create the directory "/tftpboot/pxelinux.cfg".  The

configuration file (equivalent of syslinux.cfg -- see syslinux.doc for

the options here) will live in this directory.  Because more than one

system may be booted from the same server, the configuration file name

depends on the IP address of the booting machine.  PXELINUX will

search for its config file on the boot server in the following way:

First, it will search for the config file using the hardware type

(using its ARP type code) and address, all in lower case hexadecimal

with dash separators; for example, for an Ethernet (ARP type 1)

with address 88:99:AA:BB:CC:DD it would search for the filename

01-88-99-aa-bb-cc-dd.

Next, it will search for the config file using its own IP address

in upper case hexadecimal, e.g. 192.0.2.91 -> C000025B

(you can use the included progam "gethostip" to compute the

hexadecimal IP address for any host.)

If that file is not found, it will remove one hex digit and try

again.  Ultimately, it will try looking for a file named "default"

(in lower case).

As an example, if the boot file name is /mybootdir/pxelinux.0, the

Ethernet MAC address is 88:99:AA:BB:CC:DD and the IP address

192.0.2.91, it will try:

/mybootdir/pxelinux.cfg/01-88-99-aa-bb-cc-dd

/mybootdir/pxelinux.cfg/C000025B

/mybootdir/pxelinux.cfg/C000025

/mybootdir/pxelinux.cfg/C00002

/mybootdir/pxelinux.cfg/C0000

/mybootdir/pxelinux.cfg/C000

/mybootdir/pxelinux.cfg/C00

/mybootdir/pxelinux.cfg/C0

/mybootdir/pxelinux.cfg/C

/mybootdir/pxelinux.cfg/default

... in that order.

Note that all filename references are relative to the directory

pxelinux.0 lives in.  PXELINUX generally requires that filenames

(including any relative path) are 127 characters or shorter in length.

PXELINUX does not support MTFTP, and I have no immediate plans of

doing so.  It is of course possible to use MTFTP for the initial boot,

if you have such a setup.  MTFTP server setup is beyond the scope of

this document.

++++ SETTING UP THE TFTP SERVER ++++

PXELINUX currently requires that the boot server has a TFTP server

which supports the "tsize" TFTP option (RFC 1784/RFC 2349).  The

"tftp-hpa" TFTP server, which support options, is available at:

http://www.kernel.org/pub/software/network/tftp/

ftp://www.kernel.org/pub/software/network/tftp/

... and on any kernel.org mirror (see http://www.kernel.org/mirrors/).

Another TFTP server which supports this is atftp by Jean-Pierre

Lefebvre:

ftp://ftp.mamalinux.com/pub/atftp/

If your boot server is running Windows (and you can't fix that), try

tftpd32 by Philippe Jounin (you need version 2.11 or later; previous

versions had a bug which made it incompatible with PXELINUX):

http://tftpd32.jounin.net/

++++ SETTING UP THE DHCP SERVER ++++

The PXE protocol uses a very complex set of extensions to DHCP or

BOOTP.  However, most PXE implementations -- this includes all Intel

ones version 0.99n and later -- seem to be able to boot in a

"conventional" DHCP/TFTP configuration.  Assuming you don't have to

support any very old or otherwise severely broken clients, this is

probably the best configuration unless you already have a PXE boot

server on your network.

A sample DHCP setup, using the "conventional TFTP" configuration,

would look something like the following, using ISC dhcp 2.0 dhcpd.conf

syntax:

allow booting;

allow bootp;

# Standard configuration directives...

option domain-name "<domain name>";

option subnet-mask <subnet mask>;

option broadcast-address <broadcast address>;

option domain-name-servers <dns servers>;

option routers <default router>;

# Group the PXE bootable hosts together

group {

# PXE-specific configuration directives...

next-server <TFTP server address>;

filename "/tftpboot/pxelinux.0";

# You need an entry like this for every host

# unless you're using dynamic addresses

host <hostname> {

hardware ethernet <ethernet address>;

fixed-address <hostname>;

}

}

Note that if your particular TFTP daemon runs under chroot (tftp-hpa

will do this if you specify the -s (secure) option; this is highly

recommended), you almost certainly should not include the /tftpboot

prefix in the filename statement.

If this does not work for your configuration, you probably should set

up a "PXE boot server" on port 4011 of your TFTP server; a free PXE

boot server is available at:

http://www.kano.org.uk/projects/pxe/

With such a boot server defined, your DHCP configuration should look

the same except for an "option dhcp-class-identifier" ("option

vendor-class-identifier" if you are using DHCP 3.0):

allow booting;

allow bootp;

# Standard configuration directives...

option domain-name "<domain name>";

option subnet-mask <subnet mask>;

option broadcast-address <broadcast address>;

option domain-name-servers <dns servers>;

option routers <default router>;

# Group the PXE bootable hosts together

group {

# PXE-specific configuration directives...

option dhcp-class-identifier "PXEClient";

next-server <pxe boot server address>;

# You need an entry like this for every host

# unless you're using dynamic addresses

host <hostname> {

hardware ethernet <ethernet address>;

fixed-address <hostname>;

}

}

Here, the boot file name is obtained from the PXE server.

If the "conventional TFTP" configuration doesn't work on your clients,

and setting up a PXE boot server is not an option, you can attempt the

following configuration.  It has been known to boot some

configurations correctly; however, there are no guarantees:

allow booting;

allow bootp;

# Standard configuration directives...

option domain-name "<domain name>";

option subnet-mask <subnet mask>;

option broadcast-address <broadcast address>;

option domain-name-servers <dns servers>;

option routers <default router>;

# Group the PXE bootable hosts together

group {

# PXE-specific configuration directives...

option dhcp-class-identifier "PXEClient";

option vendor-encapsulated-options 09:0f:80:00:0c:4e:65:74:77:6f:72:6b:20:62:6f:6f:74:0a:07:00:50:72:6f:6d:70:74:06:01:02:08:03:80:00:00:47:04:80:00:00:00:ff;

next-server <TFTP server>;

filename "/tftpboot/pxelinux.0";

# You need an entry like this for every host

# unless you're using dynamic addresses

host <hostname> {

hardware ethernet <ethernet address>;

fixed-address <hostname>;

}

}

Note that this *will not* boot some clients that *will* boot with the

"conventional TFTP" configuration; Intel Boot Client 3.0 and later are

known to fall into this category.

++++ SPECIAL DHCP OPTIONS ++++

PXELINUX (starting with version 1.62) supports the following

nonstandard DHCP options, which depending on your DHCP server you may

be able to use to customize the specific behaviour of PXELINUX:

Option 208	pxelinux.magic

- Must be set to F1:00:74:7E (241.0.116.126) for PXELINUX to

recognize any special DHCP options whatsoever.

Option 209	pxelinux.configfile

- Specifies the PXELINUX configuration file name.

Option 210	pxelinux.pathprefix

- Specifies the PXELINUX common path prefix, instead of

deriving it from the boot file name.  This almost certainly

needs to end in whatever character the TFTP server OS uses

as a pathname separator, e.g. slash (/) for Unix.

Option 211	pxelinux.reboottime

- Specifies, in seconds, the time to wait before reboot in the

event of TFTP failure.  0 means wait "forever" (in reality,

it waits approximately 136 years.)

ISC dhcp 3.0 supports a rather nice syntax for specifying custom

options; you can use the following syntax in dhcpd.conf if you are

running this version of dhcpd:

option space pxelinux;

option pxelinux.magic      code 208 = string;

option pxelinux.configfile code 209 = text;

option pxelinux.pathprefix code 210 = text;

option pxelinux.reboottime code 211 = unsigned integer 32;

NOTE: In earlier versions of PXELINUX, this would only work as a

"site-option-space".  Since PXELINUX 2.07, this will work both as a

"site-option-space" (unencapsulated) and as a "vendor-option-space"

(type 43 encapsulated.)  This may avoid messing with the

dhcp-parameter-request-list, as detailed below.

Then, inside your PXELINUX-booting group or class (whereever you have

the PXELINUX-related options, such as the filename option), you can

add, for example:

# Always include the following lines for all PXELINUX clients

site-option-space "pxelinux";

option pxelinux.magic f1:00:74:7e;

if exists dhcp-parameter-request-list {

# Always send the PXELINUX options (specified in hexadecimal)

option dhcp-parameter-request-list = concat(option dhcp-parameter-request-list,d0,d1,d2,d3);

}

# These lines should be customized to your setup

option pxelinux.configfile "configs/common";

option pxelinux.pathprefix "/tftpboot/pxelinux/files/";

option pxelinux.reboottime 30;

filename "/tftpboot/pxelinux/pxelinux.bin";

Note that the configfile is relative to the pathprefix: this will look

for a config file called /tftpboot/pxelinux/files/configs/common on

the TFTP server.

The "option dhcp-parameter-request-list" statement forces the DHCP

server to send the PXELINUX-specific options, even though they are not

explicitly requested.  Since the DHCP request is done before PXELINUX

is loaded, the PXE client won't know to request them.

Using ISC dhcp 3.0 you can create a lot of these strings on the fly.

For example, to use the hexadecimal form of the hardware address as

the configuration file name, you could do something like:

site-option-space "pxelinux";

option pxelinux.magic f1:00:74:7e;

if exists dhcp-parameter-request-list {

# Always send the PXELINUX options (specified in hexadecimal)

option dhcp-parameter-request-list = concat(option dhcp-parameter-request-list,d0,d1,d2,d3);

}

option pxelinux.configfile =

concat("pxelinux.cfg/", binary-to-ascii(16, 8, ":", hardware));

filename "/tftpboot/pxelinux.bin";

If you used this from a client whose Ethernet address was

58:FA:84:CF:55:0E, this would look for a configuration file named

"/tftpboot/pxelinux.cfg/1:58:fa:84:cf:55:e".

++++ ALTERNATE TFTP SERVERS ++++

PXELINUX supports the following special pathname conventions:

::filename

Suppresses the common filename prefix, i.e. passes the string

"filename" unmodified to the server.

IP address::filename		(e.g. 192.0.2.1::filename)

Suppresses the common filename prefix, *and* sends a request

to an alternate TFTP server.  Instead of an IP address, a

DNS name can be used.  It will be assumed to be fully

qualified if it contains dots; otherwise the local domain as

reported by the DHCP server (option 15) will be added.

:: was chosen because it is unlikely to conflict with operating system

usage.  However, if you happen to have an environment for which the

special treatment of :: is a problem, please contact the SYSLINUX

mailing list.

++++ SOME NOTES ++++

If the boot fails, PXELINUX (unlike SYSLINUX) will not wait forever;

rather, if it has not received any input for approximately five

minutes after displaying an error message, it will reset the machine.

This allows an unattended machine to recover in case it had bad enough

luck of trying to boot at the same time the TFTP server goes down.

Lots of PXE stacks, especially old ones, have various problems of

varying degrees of severity.  Please see:

http://syslinux.zytor.com/hardware.php

... for a list of currently known hardware problems, with workarounds

if known.

++++ KEEPING THE PXE STACK AROUND ++++

Normally, PXELINUX will unload the PXE and UNDI stacks before invoking

the kernel.  In special circumstances (for example, when using MEMDISK

to boot an operating system with an UNDI network driver) it might be

desirable to keep the PXE stack in memory.  If the option "keeppxe"

is given on the kernel command line, PXELINUX will keep the PXE and

UNDI stacks in memory.  (If you don't know what this means, you

probably don't need it.)

++++ CURRENTLY KNOWN PROBLEMS ++++

The following problems are known with PXELINUX, so far:

+ Requires a TFTP server which supports the "tsize" option.

+ The error recovery routine doesn't work quite right.  For right now,

it just does a hard reset - seems good enough.

+ We should probably call the UDP receive function in the keyboard

entry loop, so that we answer ARP requests.

+ Boot sectors/disk images are not supported yet.

If you have additional problems, please contact the SYSLINUX mailing

list (see syslinux.doc for the address.)